# Red-I™ Joist Specifications

1.0 GENERAL

1.1 Scope

This work includes the complete furnishings and installation of all Red-I™ joists, as shown on the drawings herein specified and necessary to complete the work.

1.2 Code Approvals

These products shall be designed and manufactured to the standards set forth in the International Code Council Report No. ESR-2994.

1.3 Related Work Specified Elsewhere

A.  Carpentry and Millwork

B.  Glu-Laminated Members

1.4 Design

- Products: RedBuilt™ products shall be designed to fit the dimensions and loads indicated on the plans.

- Design Calculations:  When requested, a complete set of design calculations shall be prepared by RedBuilt™.

1.5 Submittals

- Drawings: Drawings showing layout and detail necessary for determining fit and placement in the building shall be provided by RedBuilt™.

- Production: Fabrication and/or cutting shall not proceed until the architect and/or engineer have approved the submittal package.

2.0 PRODUCTS

2.1 Materials

Flange members, web members and adhesives shall conform to the provisions of ICC-ES Report No. ESR-2994.

2.2 Fabrication

Red-I™ joists shall be manufactured by RedBuilt™ in a plant listed in the report referred to above and under the supervision of an approved third-party inspection agency.

2.3 Tolerances

Depth: ± 1/16"

Flange Width: ± 1/16"

2.4 Identification

Each of the joists shall be identified by a stamp indicating the joist series, ICC-ES report number, manufacturer's name, plant number, date of fabrication, and the independent inspection agency’s logo.

2.5 Hardware

Not applicable.

3.0 EXECUTION

3.1 Installation

Red-I™ joists, if stored prior to installation, shall be protected from the weather. They shall be handled with care so they are not damaged. Red-I™ joists shall be installed in accordance with the plans, and any RedBuilt™ drawings and installation suggestions.  Temporary construction loads that cause stresses beyond design limits are not permitted. Safety bracing is to be provided by the installer to keep the Red-I™ joists straight and plumb as required, and to ensure adequate lateral support for the individual Red-I™ joist members and the entire system until the sheathing material is applied.

3.2 Installation Review

Prior to enclosing the Red-I™ joists, the Contractor shall give notification to the RedBuilt™ representative to provide an opportunity for review of the installation.

3.3 Performance Standards

Products shall be proven by testing and evaluation in accordance with the provisions of ASTM D-5055.

3.4 Fire Rating/Sound Ratings

Fire and sound ratings are to be established in accordance with the assemblies detailed in ICC-ES Report No. ESR-2994 or the Directory of Listed Products published by PFS Corporation.

3.5 Warranty

The products delivered shall be free from manufacturing errors or defects in workmanship and material.  The products, when correctly installed and maintained, shall be warranted to perform as designed for the normal and expected life of the building.

4.0 ALTERNATES AND/OR EQUALS

4.1 Base Bid

Due to the customized detailing and engineering characteristics of the roof and/or floor framing assembly, it is a requirement that RedBuilt™ Red-I™ joists be used in the base bid.

4.2 Alternate Manufacturers

Other manufacturers’ bids are to be listed in the alternate section of your proposal. All framing plans, detailing, and calculations for the alternate bids will be reviewed by the owner, architect, and engineer for structural performance, possible conflicts with related trades, and compatibility with the overall building requirements and building code.

4.3 Alternate Products

Alternate products will only be permitted if written approval and acceptance is obtained by both architect and owner at least seven (7) days prior to the bid date.  Any monetary savings that may be realized by using an alternate product shall be forwarded to the owner.

4.4 Acceptable Alternates

At the discretion of the specifier of record, accepted alternates will be listed on the final addendum prior to the bid date.